# Master's Thesis: Automated Micro-CT System for Battery Swelling Analysis

Duration: 20 weeks (January–June 2026)
Supervisor: Artem Kulachenko
Location: Engineering Mechanics, KTH

## What this project is about

Electric vehicle batteries degrade through a complex combination of mechanical and electrochemical processes. One critical degradation mode involves electrode swelling, which redistributes the electrolyte within the cell, creating salt concentration gradients and ultimately leading to lithium plating and capacity loss. Understanding these mechanisms requires tracking internal structural changes during battery operation, but current methods are either destructive or lack spatial resolution.

This thesis aims to develop an automated system that combines our Nikon micro-CT scanner (with a resolution of 1 μm) with electrochemical cycling equipment to measure three-dimensional swelling and structural changes in coin cells during charge-discharge cycles. The focus is on building reliable hardware, developing control software for autonomous operation, and establishing validated protocols that can run unattended overnight while collecting high-quality data.

Your task is to design and build the apparatus, write the control code that coordinates the potentiostat with the CT scanner, implement safety monitoring, and use the system to test several cells and extract reproducible swelling metrics.

## Technical scope

You will work with commercial graphite-NMC coin cells (CR2032 format). The Nikon XT H 225 ST 2x scanner can achieve a resolution of 1-3 μm, but at this scale, the field of view is limited, and each tomogram takes 1-2 hours to complete. We can resolve layer thickness changes, larger voids, and measure strain through image registration; however, fine features, such as the solid-electrolyte interface or nanoscale pores, remain below our detection limits. The conclusions will be on the meso-scale.

The system must operate safely without thermal control inside the CT cabinet. Temperature will be monitored using thermocouples, and the protocol will be aborted if the drift exceeds ±2°C. Safety interlocks will enforce voltage limits (2.5-4.1 V for NMC cells), verify current cutoff before scans, and provide emergency stop capability. You will need to design a windowed coin cell holder with electrical feedthroughs, integrate force and temperature sensors, and wire everything to a control computer that talks to both the BioLogic SP-50 potentiostat and the CT system.

## What you will build

1) Hardware.

A custom coin cell holder designed in CAD with a Kapton window for X-ray transparency, thin current collectors, integrated sensors for temperature and force, and proper electrical isolation. The holder mounts on the CT rotation stage, featuring feedthroughs that allow for wiring without tangling. The budget is 30,000 SEK for machining, sensors, and small components.

2) Control software.

A Python or MATLAB program that implements the test protocol: initialise the cell, cycle to a target state of charge, pause and wait for thermal stabilisation, trigger the CT scan, wait for completion, resume cycling, and repeat. The code must interface with the BioLogic API for electrochemical control, log all data with synchronised timestamps, monitor safety limits in real-time, and send email alerts if anything goes wrong. You should also build a simple dashboard to visualise cell voltage, temperature, and scan progress during operation.

3) Analysis pipeline (ideally).

Automated processing of CT data using Simpleware and Python scripts. This includes reconstruction, artefact correction (beam hardening, ring artefacts), image registration to track deformation, segmentation to measure layer thickness and void volume, and metric extraction. The goal is to feed raw scans into one end and obtain swelling curves and strain maps at the other end with minimal manual intervention.

4) Documentation.

Complete assembly instructions with CAD files, software documentation including examples and a troubleshooting guide, standard operating procedures for safe cell testing, and validation data demonstrating the system's reliable operation.

## Timeline and milestones

The 20-week schedule is tight but manageable if you stay focused. Weeks 1-3 cover system design, safety specification, and component ordering. Weeks 4-6 are hardware fabrication and assembly, with a critical decision gate at week 6: if the holder is not validated by then, we simplify the scope to manual operation with reduced automation. Weeks 7-10 focus on control software development and system integration testing with dummy cells. Weeks 11-14 are dedicated to pilot experiments with real cells, aimed at validating protocols and refining analysis workflows. Weeks 15-18 run the main experimental campaign with at least three cells to generate publishable data. Weeks 19-20 are for final analysis, thesis writing, and preparing open-source releases.

You should plan to work systematically and document as you go. The thesis can be written incrementally rather than all at once, which reduces stress at the end.